Solution and Answer Guide

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,  © 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

Chapter 4

4-1. Define

Answers:

*(a) sample standard deviation.

The sample standard deviation is the standard deviation of a sample of data. It  applies to a small set of data from the population. It uses deviations from the experimental mean and the number of degrees or freedom N − 1 in place of the  population mean μ and N.

(b) coefficient of variation.

The coefficient of variation is the percent relative standard deviation or ( s x/ ) ⋅ 100%.

*(c) variance.

The variance is the square of the standard deviation.

(d) standard error of the mean.

The standard error of the mean is the standard deviation of the mean and is given  by the standard deviation of the data set divided by the square root of the number  of measurements.

4-2. Differentiate between

Answers:

*(a) parameter and statistic.

The term parameter refers to quantities such as the mean and standard deviation  of a population or distribution of data. The term statistic refers to an estimate of a  parameter that is made from a sample of data.

(b) population mean and sample mean.

The population mean is the true mean for the population of data. The sample mean is the arithmetic average of a limited sample drawn from the population.

*Answers and solutions are provided in the Student Solutions Manual for questions and problems marked with an asterisk.

1 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

*(c) random and systematic error.

Random errors result from uncontrolled variables in an experiment while systematic  errors are those that can be ascribed to a particular cause and can usually be  determined.

(d) accuracy and precision.

Accuracy represents the agreement between an experimentally measured value and the true or accepted value. Precision describes the agreement among measurements that have been performed in exactly the same way.

4-3. Distinguish between

Answers:

*(a) the sample variance and the population variance.

The sample variance s2 is the variance of a sample drawn from the population. It is  ∑ ) N( x x − 2

i

given by

s2 = i=1 , N −1 where x is the sample mean.

The population variance σ2 is the variance of an entire population given by

∑ ) N( x − μ 2

i

σ2 = i=1 , N where μ is the population mean.

(b) the meaning of the word sample as it is used in a chemical and in a statistical sense.

In the chemical sense, a sample is what is taken for chemical analysis. In statistics a sample is a subset taken from the population used to infer information about the population.

4-4. What is the standard error of a mean? Why is the standard deviation of the mean lower than the standard deviation of the data points in a set?

Solution:

The standard error of a mean, sm, is the standard deviation of the set of data, s, divided  by the square root of the number of data in the set, i.e. s s m = / N. The standard error  of the mean sm is lower than the standard deviation of the data points in a set s because  a set of data made up of means will have less spread than a set of data made up of data  points. In the equations for sm, the denominator ( ) N always has a value greater than 1 so that sm will always be less than s.

2 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

*4-5. From the Gaussian (normal) error curve, what is the probability that a result from a  population lies between 0 and +1σ of the mean? What is the probability of a result  occurring that is between +1σ and +2σ of the mean?

Answer:

Since the probability that a result lies between −1σ and +1σ is 0.683, the probability that  a result will lie between 0 and +1σ will be half this value or 0.342. The probability that a  result will lie between +1σ and +2σ will be half the difference between the probability of  the result being between −2σ and +2σ, and −1σ and +1σ, or ½ (0.954−0.683) = 0.136.

4-6. From the normal curve of error, find the probability that a result is outside the limits of  ±2σ from the mean. What is the probability that a result has a more negative deviation  from the mean than −2σ?

Answer:

Since the probability that a result lies between −2σ and +2σ is 0.954, the probability that  a result will lie outside this range is (1 − 0.954) = 0.046. The probability that a result will  be more negative than −2σ will be half this value, or 0.023.

4-7. Consider the following sets of replicate measurements:

*A B *C D *E F

9.5 55.35 0.612 5.7 20.63 0.972

8.5 55.32 0.592 4.2 20.65 0.943

9.1 55.20 0.694 5.6 20.64 0.986

9.3 0.700 4.8 20.51 0.937

9.1 5.0 0.954

For each set, calculate the (a) mean, (b) median, (c) spread or range, (d) standard  deviation, and (e) coefficient of variation.

Solutions:

xi x2i

9.5 90.25

8.5 72.25

9.1 82.81

9.3 86.49

9.1 82.81

Σxi = 45.5 Σx 2i = 414.61

mean: x = 45.5/5 = 9.1

median = 9.1

spread: w = 9.5 − 8.5 = 1.0

standard deviation: = 414.61− ( ) 45.5 2/ 5

s = 0.37

5 1−

coefficient of variation: CV = (0.37/9.1) ⋅ 100% = 4.1%

3 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

Results for Sets A through F, obtained in a similar way, are given in the following table.

A B C D E F (a) x 9.1 55.29 0.650 5.1 20.61 0.958 (b) median 9.1 55.32 0.653 5.0 20.64 0.954 (c) w 1.0 0.15 0.108 1.5 0.14 0.049 (d) s 0.37 0.08 0.056 0.6 0.07 0.02 (e) CV, % 4.1 0.14 8.5 12.2 0.32 2.1

4-8. The accepted values for the sets of data in Problem 4-7 are *set A, 9.0; set B, 55.33; *set  C, 0.630; set D, 5.4; *set E, 20.58; and set F, 0.965. For the mean of each set, calculate (a)  the absolute error and (b) the relative error in parts per thousand.

Solutions:

For Set A, E = 9.1 − 9.0 = 0.1

Er = (0.1/9.0) ⋅ 1000 ppt = 11.1 ppt

Set B, E = −0.040 Er = −0.7 ppt

Set C, E = 0.0195 Er = 31 ppt

Set D, E = −0.34 Er = −63 ppt

Set E, E = 0.03 Er = 1.3 ppt

Set F, E = −0.007 Er = −6.8 ppt

4-9. Estimate the absolute deviation and the coefficient of variation for the results of the  following calculations. Round each result so that it contains only significant digits. The  numbers in parentheses are absolute standard deviations.

Solutions:

*(a) y = 3 9. 5( ) ± . 0 03 + 0 9. 93(± . 0 001)

2 2 7. ± 025( ) 0.001 = 2.082

s = + ( ) 0.03 2 2

y (0.001) + (0.001)2= 0.030

CV = (0.03/−2.082) ⋅ 100% = −1.4%

y = −2.08(±0.03)

(b) y = 15.57( ) ± . 0 04 + 0.0037(± . 0 0001)

13. ± 59( ) 0.08 = 19.1637

sy = + ( ) 0.04 2 2 (0.0001) + (0.08)2= 0.089

CV = (0.089/19.1637) ⋅ 100% = 0.46%

y = 19.16(±0.09)

4 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

*(c) y = . 29 2( ) ±0.3 ⋅ 2.03(±0.02) ⋅10217

= . 5 93928⋅10216

s ⎛ ⎞ 0.3 2⎛ ⎞2

0.02 ⋅10−17 y = + ⎜ ⎟ ⎜ ⎟ − =

⎝ ⎠ ⎝ ⎠ ⋅ 17 0.01422

y 29.2 2.034 10

CV = (0.0142) ⋅ 100% = 1.42%

sy = (0.0142) ⋅ (5.93928 ⋅ 10−16) = 0.08446 ⋅ 10−16

y = 5.94(±0.08) ⋅ 10−16

(d)74 ( ) ±

= ± 6( ) 0 2

y 32 1 ⋅1. ± 964( ) 0.006

= . 122,830 9572

sy ⎛ ⎞ 1 2 2 2 ⎛ ⎞⎛ 0.006 ⎞2

= + ⎜ ⎟⎜ ⎟ + ⎜ ⎟ = 0.00510

y ⎝ ⎠ 326 ⎝ ⎠ 740 ⎝ 1.964 ⎠

CV = (0.00510) ⋅ 100% = 0.510%

sy = (0.00510) ⋅ (122830.9572) = 626

y = 1.228(±0.006) ⋅ 105

187( ) ± − 6 89(±3) y = = () ( ) 7 5. 559 1022

*(e) ⋅

1240 ± + 1 57 ±8

s 2 2

num = + ( ) 6 ( ) 3 = 6.71 ynum = 187 − 89 = 98

s = + ( )2 2

den 1 (8) = 8.06 yden = 1240 + 57 = 1297

sy ⎛ ⎞ 6.71 2 2 ⎛ 8.06 ⎞

= + ⎜ ⎟⎜ ⎟ = 0.0688

y ⎝ ⎠ 98 ⎝1297 ⎠

CV = (0.0688) ⋅ 100% = 6.88%

sy = (0.0688) ⋅ (0.075559) = 0.00520

y = 7.6(±0.5) ⋅ 10−2

(f)3 5. ± 6( ) 0.01

y = = ( ) 6.81992 ⋅1023

522 ±3

sy ⎛ ⎞ 0.01 2 2 ⎛ 3 ⎞

= + ⎜ ⎟⎜ ⎟ = 0.006397

y ⎝ ⎠ 3.56 ⎝ 522 ⎠

CV = (0.006397) ⋅ 100% = 0.6397%

sy = (0.006397) ⋅ (6.81992 ⋅ 10−3) = 4.36 ⋅ 10−5

y = 6.82(±0.04) ⋅ 10−3

5 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

4-10. Estimate the absolute standard deviation and the coefficient of variation for the results  of the following calculations. Round each result to include only significant figures. The  numbers in parentheses are absolute standard deviations.

Solutions:

*(a) 29 y = . 1 02( ) ±0 0. 2 ⋅1028 − 3 5. 4(±0 2. ) ⋅10

2 2

s = ⋅ ( ) − − 8 9

y 0.02 10 + (0.2⋅10 ) = 2.83⋅10−10

y = 1.02⋅10−8 − 3.54⋅10−9 = 6.66⋅10−9

2.83⋅10−10

CV = −9 ⋅ = 100% 4.25%

6.66 ⋅10

y = 6.7 ±0.3 ⋅ 10−9

(b) y = 90.31( ) ± . 0 08 − 89.32(± . 0 06)

+ . 0 200( ) ±0.004

s ( ) 08 2 2

y = + 0. (0.06) + (0.004)2= 0.10

y = 90.31 − 89.32 + 0.200 = 1.190

0.10 CV = ⋅ = 100% 8.41%

1.190

y = 1.2(±0.1)

*(c) y = 0.0040( ) () ± . 0 0005 ⋅10.28 ± . 0 02

⋅ ± 347( ) 1

sy ⎛ ⎞ 0.0005 2 2 ⎛ 0.02 ⎞ ⎛ 1 ⎞

= + ⎜ ⎟⎜ ⎟ + ⎜ ⎟ = 0.1250

y ⎝ ⎠ 0.0040 ⎝10.28 ⎠ ⎝ 347 ⎠

CV = (0.1250) ⋅ 100% = 12.5%

y = 0.0040 ⋅ 10.28 ⋅ 347 = 14.27

sy = (0.125) ⋅ (14.27) = 1.78

y = 14(±2)

(d)223( ) ± . 0 03 ⋅10214

y =1. ± 47( ) 0.04 ⋅10216

s ⎛ ⎞2 2

y 0.03 ⋅ ⋅ 10− − 14 ⎛ 0.04 10 16 ⎞

= + ⎜ ⎟ − − 14 ⎜ 16 ⎟ = 0.0272

y ⎝ ⎠ 223 ⋅ ⋅ 10 ⎝ 1.47 10 ⎠

CV = (0.027) ⋅ 100% = 2.7%

y = 223⋅10−14 / 1.47⋅10−16 = 1.517 ⋅ 104

sy = (0.0272) ⋅ (1.517 ⋅ 104) = 0.0413 ⋅ 104

y = 1.52(±0.04) ⋅ 104

6 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

*(e)100( ) ±1

y = 2 1 ( ) ±

sy ⎛ ⎞ 1 1 2 2 ⎛ ⎞

= + ⎜ ⎟ ⎜⎟ = 0.500

y ⎝ ⎠ 100 ⎝ 2 ⎠

CV = (0.500) ⋅ 100% = 50.0%

y = 100 / 2 = 50.0

sy = (0.500) ⋅ (50.0) = 25

y = 50(±25)

(f)1. ± 49( ) 0.02 ⋅102 2 2 3 − 4. ± 97( ) 0.06 ⋅10 y = 27. ± 1 0 () ( .7 + 8.99 ±0.08)

( )2 2

s = ⋅ − − 2 3

−4

num 0.02 10 + (0.06 ⋅ 10 ) = 2.09 ⋅ 10

num = 0.0149 − 0.00497 = 0.00993

s =+= ( )2 2

den 0.7 (0.08) 0.704

den = 27.1 + 8.99 = 36.09

s ⎛ ⎞ 0.000209 2 2

y ⎛ 0.704 ⎞ = + ⎜ ⎟⎜ ⎟ = 0.0287

y ⎝ ⎠ 0.00993 ⎝ 36.09 ⎠

CV = (0.0287) ⋅ 100% = 2.87%

y = 0.00993/36.09 = 2.751 ⋅ 10−4

sy = (0.0287) ⋅ (2.751 ⋅ 10−4) = 7.895 ⋅ 10−6  y = 2.75(±0.08) ⋅ 10−4

4-11. Calculate the absolute standard deviation and the coefficient of variation for the results  of the following calculations. Round each result to include only significant figures. The  numbers in parentheses are absolute standard deviations.

Solutions:

*(a) ⎤⎦ y = . log ⎡2 00( ) ±0.03 ⋅1024

⎣

y = log(2.00 ⋅ 10−4) = −3.6989 ( ) 0.434 ( ) 0.03 ⋅10−4

s =( ) 6.51 10−3

y2.00 ⋅10−4 = ⋅

y = −3.699 ±0.006

CV = (0.006/3.699) ⋅ 100% = 0.16%

(b) ⎤⎦ y = . log ⎡4 42( ) ±0.01 ⋅1037 ⎣

As in part (a): y = 37.645 ±0.001

CV = 0.003%

7 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

*(c) . )⎤⎦ y = . antilog ⎡1 200(±0 003 ⎣

y = antilog(1.200) = 15.849 sy = = ( ) 2.303 (0.003) 0.0069

y

sy = (0.0069)(15.849) = 0.11

CV = (0.11/15.8) ⋅ 100% = 0.69%

(d) ⎤⎦ y = . antilog ⎡49 54( ) ±0.04 ⎣ As in part (c): y = 3.5(±0.3) ⋅ 1049 CV = 9.2%

y = 15.8 ±0.1

4-12. Calculate the absolute standard deviation and the coefficient of variation for the results  of the following calculations. Round each result to include only significant figures. The  numbers in parentheses are absolute standard deviations.

Solutions:

*(a) ⎤⎦3

y = . ⎡4 17( ) ±0.03 ⋅1024

⎣

sy ⎛ ⎞ 0.03⋅10−4

y = (4.17 ⋅ 10−4)3 = 7.251 ⋅ 10−11 .0216

= = 3 0 ⎜ ⎟

y ⎝ ⎠ 4.17 ⋅10−4

= (0.0216)(7.251 ⋅ 10−11) = 1.565 ⋅ 10−12 y = 7.3(±0.2) ⋅ 10−11

sy

CV = (1.565 ⋅ 10−12/7.251 ⋅ 10−11) ⋅ 100% = 2.2%

(b) ⎤⎦ y 2 1 4

= . ⎡ 936( ) ±0.002 /

⎣

As in part (a): y = 1.3090(±0.0002)

CV = 0.02%

*4-13. The standard deviation in measuring the diameter d of a sphere is ±0.02 cm. What is the  standard deviation in the calculated volume V of the sphere if d = 2.35 cm?

Solution:

From the equation for the volume of a sphere, we have

4 4 3 ⎛ ⎞ d 3 3 4 ⎛ 2.35 ⎞ V r = = π π 3⎜ ⎟ = π ⎜ ⎟ = 6.80 cm

3 3 ⎝ ⎠ 2 3 ⎝ 2 ⎠

Hence, we may write

sV sd 0.02

= ⋅ 3 = 3 ⋅ = 0.0255

V d 2.35

sV = 6.80 ⋅ 0.0255 = 0.173

V = 6.8(±0.2) cm3

8 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

4-14. The inside diameter of an open cylindrical tank was measured. The results of four  replicate measurements were 5.2, 5.7, 5.3, and 5.5 m. Measurements of the height of the  tank yielded 7.9, 7.8, and 7.6 m. Calculate the volume in liters of the tank and the  standard deviation of the result.

Solution:

The mean diameter of the tank is 5.2 + + 5.7 5.3 + 5.5 d = = 5.425 m 4

∑N( ) x x − 2

i

s = i=1

The standard deviation of the diameter is

d = 0.222  N −1

The mean height of the tank is 7.9 + + 7.8 7.6 h = = 7.767 m and s

3 h = 0.153

The volume of the tank is given by

222 2

⎛ ⎞ d 2 ⎛ ⎞ 5.425( ) ±0.

V h = ⋅ π π ⎜ ⎟ = 7.767(±0.153) ⋅ ⋅ ⎜ ⎟

⎝ ⎠ 2 ⎝ ⎠ 2

The error in the 3rd factor is given by

s ⎛ ⎞ 0 2

y .222 = = ⎜ ⎟ 0.0409 y = ½ (5.425) = 2.7125 sy = 0.0409 ⋅ 2.7125 = 0.111  y ⎝ ⎠ 5.425

V = 7.767(±0.153) ⋅ (3.14159) ⋅ (2.7125(±0.111))2

sy ⎛ ⎞ 0.111 = = 2⎜ ⎟ 0.0818 y = (2.7125)2 = 7.358 s

y 2.7125 y = (0.0818)(7.358) = 0.602 ⎝ ⎠  V = 7.767(±0.153) ⋅ (3.14159) ⋅ 7.358(±0.602)

Next, we propagate the error in volume by assuming the error in pi is negligible.  sy ⎛ ⎞ 0.153 2 2 ⎛ ⎞ 0.602

= + ⎜ ⎟⎜ ⎟ = 0.0841 y = 7.767 ⋅ 3.14159 ⋅ 7.358 = 179.5

y ⎝ ⎠ 7.767 ⎝ ⎠ 7.358

sy = (0.0841)(179.5) = 15.09

y = 180(±20) m3

Converting to liters, we have

10 V = 180(±20) m3 00L

⋅m3 = 1.8(±0.2) ⋅ 105 L

*4-15. In a volumetric determination of an analyte A, the data obtained and their standard  deviations are as follows:

Initial buret reading 0.19 mL 0.02 mL

Final buret reading 9.26 mL 0.03 mL

Sample mass 45.0 mg 0.2 mg

9 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

From the data, find the coefficient of variation of the final result for the %A that is  obtained by using the equation that follows and assuming there is no uncertainty in the  equivalent mass.

%A = titrant volume ⋅ equivalent mass ⋅ 100%/sample mass

Solution:

Since the titrant volume equals the final buret reading minus the initial buret reading, we  can introduce the values given into the equation for %A.

%A = [9.26(±0.03) − 0.19(±0.02)] ⋅ equivalent weight ⋅ 100/[45.0(±0.2)]

Obtaining the value of the first factor and the error in the first factor

s = + ( ) 0.03 2 2

y (0.02) = 0.0361 y = 9.26 − 0.19 = 9.07

We can now obtain the relative error of the calculation

s 0.036 2 2

%A ⎛ ⎞⎛ 0.2 ⎞

= + ⎜ ⎟⎜ ⎟ = 0.00596

%A ⎝ ⎠ 9.07 ⎝ 45.0 ⎠

The coefficient of variation is then

CV = (0.00596) ⋅ 100% = 0.596% or 0.6%

4-16. Chapter 26 describes inductively coupled plasma atomic emission spectrometry. In that  method, the number of atoms excited to a particular energy level is a strong function of  temperature. For an element of excitation energy E in joules (J), the measured ICP  emission signal S can be written as

S k = ′e2E / kT

where k′ is a constant independent of temperature, T is the absolute temperature in  kelvin (K), and k is Boltzmann’s constant (1.3807 ⋅ 10−23 J K−1). For an ICP of average  temperature 7000 K and for Cu with an excitation energy of 6.12 ⋅ 10−19 J, how precisely  does the ICP temperature need to be controlled for the coefficient of variation in the  emission signal to be 1% or less?

Solution:

To obtain a CV in S of 1% or less,

e s 2 2

S ⎛ ⎞ s ⎛ ⎞ s

≤ = 0.01 ⎜ ⎟ k′ + ⎜ ⎟ −E k/ T

S k′ ⎜ ⎟ ⎝ ⎠ e−E k/ T

⎝ ⎠

Since k′ is a constant, the first term is zero resulting in:

e−E k/ T From Table 4-4, we find se−E k/ T

s

0.01= e−E k/ T

−E k/ T = s

e −E k/ T

10 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

s−E/kT can be determined by evaluation of the errors in each of the numbers.  s s 222

−E k/ T ⎛⎞ ⎛⎞ ⎛⎞

= + ⎜ ⎟ E s⎜ k s ⎟ + ⎜ T ⎟ −E k/ T ⎝ ⎠ E ⎝ k ⎠ ⎝ T ⎠

Since both E and k have no uncertainty, this equation reduces to:

⎛ ⎞ ⎛ ⎞ s s −

T T − − E ⎛ ⎞ 6.12 ⋅10 19

s−E k = ⋅= ⎜ ⎟ / T ⎜⎟ ⎜ ⎟ ( ) = 0.01 ⎜ ⎟ ⎝ ⎠ T kT ⎝ 7000 ⎠ 1.3807 ⋅10−23 ( ) 7000 ⎝ ⎠

Solving for sT gives:

sT ≤ 11.1 K

*4-17. Chapter 22 shows that quantitative molecular absorption spectrometry is based on  Beer’s law, which can be written as

2log = T bε cX

where T is the transmittance of a solution of an analyte X, b is the thickness of the  absorbing solution, c X is the molar concentration of X, and ε is an experimentally

determined constant. By measuring a series of standard solutions of X, εb was found to  have a value of 3312(±12) M−1, where the number in parentheses is the absolute  standard deviation.

An unknown solution of X was measured in a cell identical to the one used to  determine εb. The replicate results were T = 0.213, 0.216, 0.208, and 0.214. Calculate (a)  the molar concentration of the analyte cX, (b) the absolute standard deviation of the cX,  and (c) the coefficient of variation of cX.

Solutions:

We first calculate the mean transmittance and the standard deviation of the mean.

⎛ 0.213 +++ 0.216 0.208 0.214 ⎞⎟⎠

mean T = ⎜ = 0.2128

⎝ 4

sT = 0.0034

(a) ⎛ ⎞ −logT −log( ) 0.2128

cX = ⎜ ⎟ = = 2.029⋅10−4 M

⎝ ⎠ εb 3312

(b) For −logT, sy = (0.434)sT/T = 0.434 ⋅ (0.0034/0.2128) = 0.00693

−log(0.2128) = 0.672

− ± logT 0.672 0.00693

cX = = εb 3312 ± 12

s 2 2

CX ⎛ ⎞ 0.00693 ⎛ 12 ⎞

= + ⎜ ⎟⎜ ⎟ = 0.0109

cX ⎝ ⎠ 0.672 ⎝ 3312 ⎠

s X ( ) 0.0109 ( ) 4 6

C = ⋅ 2.029 10− − = 2.22 ⋅ 10

(c) CV = (2.22 ⋅ 10−6/2.029 ⋅ 10−4) ⋅ 100% = 1.1%

11 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

4-18. Analysis of several plant-food preparations for potassium ion yielded the following data:  Sample Percent K+

1 6.02, 6.04, 5.88, 6.06, 5.82

2 7.48, 7.47, 7.29

3 3.90, 3.96, 4.16, 3.96

4 4.48, 4.65, 4.68, 4.42

5 5.29, 5.13, 5.14, 5.28, 5.20

The preparations were randomly drawn from the same population.

(a) Find the mean and standard deviation s for each sample.

(b) Obtain the pooled value spooled.

Solutions:

(a) and (b)

(c) Why is spooled a better estimate of σ than the standard deviation from any one sample?  Answer:

Pooling the variations in %K from the five samples gives a better estimate of σ because several data sets are used. Thus the pooled standard deviation uses a  larger number of data points and can reflect variations that arise because of  sample selection and sample preparation.

12 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

*4-19. Six bottles of wine of the same variety were analyzed for residual sugar content with the  following results:

Bottle Percent (w/v) Residual Sugar

1 1.02, 0.84, 0.99

2 1.13, 1.02, 1.17, 1.02

3 1.12, 1.32, 1.13, 1.20, 1.25

4 0.77, 0.58, 0.61, 0.72

5 0.73, 0.92, 0.90

6 0.73, 0.88, 0.72, 0.70

(a) Evaluate the standard deviation s for each set of data.

Solution:

The standard deviations are s1 = 0.096, s2 = 0.077, s3 = 0.084, s4 = 0.090, s5 = 0.104, s6 = 0.083

(b) Pool the data to obtain an absolute standard deviation for the method.

Solution:

spooled = 0.088 or 0.09

13 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

4-20. Nine samples of illicit heroin preparations were analyzed in duplicate by a gas  chromatographic method. The samples can be assumed to have been drawn randomly from  the same population. Pool the following data to establish an estimate of σ for the procedure.

Sample Heroin, % Sample Heroin, %

1 2.24, 2.27 6 1.07, 1.02

2 8.4, 8.7 7 14.4, 14.8

3 7.6, 7.5 8 21.9, 21.1

4 11.9, 12.6 9 8.8, 8.4

5 4.3, 4.2

Solution:

14 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

*4-21.

Calculate a pooled estimate of σ from the following spectrophotometric analysis for NTA  (nitrilotriacetic acid) in water from the Ohio River:

Sample NTA, ppb

1 13, 19, 12, 7

2 42, 40, 39

3 29, 25, 26, 23, 30

Solution:

15 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.

Solution and Answer Guide: Skoog et al., Fundamentals of Analytical Chemistry, 10e,

© 2022, 978-0-357-45039-0, Chapter 4: Random Errors in Chemical Analysis

4-22. Go to http://dx.doi.org/10.18434/T43G6C, where you will find a link to the NIST  Statistical Reference Datasets site. Find the Dataset Archives, locate the Univariate  Summary Statistics section, and select the Mavro data set. This actual data set is the  result of a study by NIST chemist Radu Mavrodineaunu. His study was to determine a  certified transmittance value for an optical filter. Click on the ASCII format data file. The  data set at the bottom of the page contains 50 transmittance values collected by  Mavrodineaunu. Once you have the data on the screen, use your mouse to highlight only  the 50 transmittance values, and click on Edit/Copy (or use Ctrl-C) to place the data on  the clipboard. Then start Excel with a clean spreadsheet, and click on Edit/Paste (Ctrl-V)  to insert the data in column B. Now, find the mean and standard deviation, and compare  your values to those presented when you click on Certified Values on the NIST web page.  Be sure to increase the displayed number of digits in your spreadsheet so that you can  compare all the digits. Comment on any differences between your results and the  certified values. Suggest possible sources for the differences.

Solution:

Excel value NIST value

Mean 2.001856000000000 2.001856000000000

SD 0.000429123454003085 0.000429123454003053

The means are the same in both cases. There is some difference, however, in the  standard deviations in the last two digits. The differences could arise because of the  different algorithms used to calculate the SD and to roundoff errors in the calculations.  The first 16 digits are identical.

16 © 2022 Cengage. All Rights Reserved. May not be scanned, copied or duplicated, or posted to a  publicly accessible website, in whole or in part.